# Activity 5.4: Other Organic Materials Reading

# Propane, and Propane Combustion

If you have ever seen a gas grill or a large steel tank outside of your home you probably have heard of propane, but what is it?  You can’t see through the steel tank, so you cannot see what it looks like.

The energy in propane starts out as chemical energy in the molecule of propane.  This reading answers two questions:

- What is propane?

- How does a propane grill work?

## What is propane?

Propane is an organic molecule that has the chemical structure: C3H8.  It is a hydrocarbon: a molecule made up of only carbon and hydrogen.  Here is a diagram of the structure of a single propane molecule.  Propane is a colorless gas at normal atmospheric pressure, but when it is compressed it condenses and becomes a liquid.

Since it has all C-C and C-H bonds, propane is a good fuel. Why does that make it a good fuel?  Propane is also a fossil fuel: It was formed by the decay of plants and animals millions of years ago, and now we can pump it up from underground as one component of petroleum and natural gas—both mixtures of hydrocarbons and other molecules.  The petroleum and natural gas are sent to refineries—processing plants that separate them into different kinds of molecules, including propane.

Since propane is a colorless gas, it’s hard to see.  But it’s easy to see tanks full of propane that we use for fuel like those below.  Gas grills use small propane tanks.  Some houses or businesses have large propane containers that provide fuel for heating and cooking. These containers hold propane under pressure. This pressure on the gas turns it into a liquid for storage.

Figure 1  A 20 lb (9.1 kg) steel propane cylinder” by Hustvedt is licensed under CC BY 3.0

Figure 2 11,000 L Fuel Tank” by Westeel1 is licensed under CC BY 4.0

## How does a propane grill work?

One of the most common uses of propane is for cooking foods on gas grills. Gas grills turn the chemical energy in propane into heat for cooking. Let’s start by looking at the structure and function of a gas grill.

Gas grills have lots of parts. The diagram below shows just the most important ones.  The grill has:

A propane tank

Hoses that carry propane gas to the burners

Burner tubes that mix propane with air, then release it to be burned

Figure 3 Basic parts of a gas grill

The arrows in Figure 4 show how propane gets to the burners. So, here’s a puzzle. If the propane liquid is in the bottom of the tank, why is the valve that releases propane at the top of the tank? Putting the valve at the top of the tank is actually a clever design. Propane is a gas at normal atmospheric pressure, but when compressed, it is a liquid. Therefore, when the propane gas goes into the hose from the valve, the pressure in the tank goes down, and some of the propane liquid evaporates. So as long as there is any liquid to evaporate, the tank stays full of propane gas under pressure.

Figure 4: Air and propane gas entering the burner tube

## What happens to propane when it burns?

Now let’s look more closely at what happens when propane reaches the burner tubes.

Propane can burn hotter and cleaner if it is mixed with air before it starts to burn, so propane burners do that. Then the propane and air mixture leaves holes in the burner tube. The spark that starts them burning is outside the tube.

You already know some parts of the story of propane burning. It can help to think about how burning propane is alike and different from burning other organic materials that you have studied, such as ethanol or methane.

Alike: Like ethanol and methane, propane is an organic material, with lots of high-energy C-C and C-H bonds than can release energy when they are combined with oxygen (O2). Propane, ethanol, and methane are also alike in that they enter the flame as gases (after evaporating from liquids in the case of ethanol and propane).

Different: Propane molecules have a different structure from ethanol and methane, C3H8.  So the chemical equation for burning propane looks something like this:

C3H8 + 5 O2 = 3 CO2 + 4 H2O

Use the worksheet to tell the story based on what you know about propane and about combustion of other organic materials.

## Digging deeper

Here are some more places that you can go to learn about where propane comes from, what is in it, and how combustion works:

Read more about where propane comes from and what it is:

  - University of Colorado Article, Hyperlink

Learn more about how propane tanks work:

  - How propane tanks work, hyperlink

Learn more about where propane comes from and how scientists are working on figuring out renewable ways to make propane:

  - Renewable ways to form propane, hyperlink